DB tool(ALiEditor) User Manual

- Enter the [REC] Menu and select the [Upgrade By USB] item.

- Select the [Backup channel] mode, and click the [Start] item.

- Copy the backup channel file (“Channel200911272100.bin”) to your computer.

- Rename  “Channel200911272100.bin” to “Channel200911272100.udf”

- Open the “AliEditor.exe”. Select the menu or Tool bar to click the Open item. Open the “Channel200911272100.udf”.

- The “Left window” list the Satellite/TP tree/Favorite Group. When select one TP, The “Right Window” will list the Programs for this TP.

- Add Satellite.

- Edit Satellite.

Note: The Satellite [Tuner] item must be [Tuner1_alid].

- Add TP.

- Edit TP.

- Add Program.

- Edit Program.

Note: Should first select one TP, then user can Insert Channel.

- Select the menu or Tool bar to click the Save item after editing.

- Select the menu or Tool bar to click the Export item.

- Rename the “Channel200911272100.ddf” to “NEWChannel.bin”, and copy it to your U disk.

- Enter the [REC] Menu and select the [Upgrade By USB] item.

- Select the [defaultdb] mode, and select the “NEWChannel.bin” file to update defaultdb.

- Enter the [Installation] Menu and select the [Factory Setting] to take effect.